# Chapter 1: Introduction to Digital Image Processing - MCQs

## Q1: What is a digital image defined as?

A) A continuous one-dimensional function

B) A two-dimensional function f(x,y) with finite, discrete quantities

C) An analog spectrum

D) A continuous three-dimensional wave

Correct Answer: B) A two-dimensional function f(x,y) with finite, discrete quantities

Explanation (EN): A digital image is defined as a two-dimensional function f(x,y) where the coordinates and intensity values are finite, discrete quantities.

الشرح (AR): تُعرف الصورة الرقمية كدالة ثنائية الأبعاد f(x,y) حيث تكون الإحداثيات وقيم الشدة كميات منتهية ومنفصلة.

________________________________________

## Q2: Which of the following terms is NOT used to denote the elements of a digital image?

A) Picture elements

B) Image elements

C) Pels

D) Continuous variables

Correct Answer: D) Continuous variables

Explanation (EN): The elements of a digital image are called picture elements, image elements, pels, and pixels. They are discrete, not continuous variables.

الشرح (AR): عناصر الصورة الرقمية تسمى بكسلات (pixels)، أو pels، وليست متغيرات مستمرة.

________________________________________

## Q3: Imaging machines cover almost the entire electromagnetic (EM) spectrum, ranging from:

A) Visual band to infrared

B) Gamma to radio waves

C) X-rays to microwaves

D) Ultraviolet to acoustic waves

Correct Answer: B) Gamma to radio waves

Explanation (EN): Unlike humans who are limited to the visual band, imaging machines cover almost the entire EM spectrum from gamma to radio waves.

الشرح (AR): على عكس البشر المحدودين بالطيف المرئي، تغطي آلات التصوير الطيف الكهرومغناطيسي بالكامل تقريبًا من أشعة غاما إلى موجات الراديو.

________________________________________

## Q4: What characterizes a low-level image processing operation?

A) Input is an image, output is an attribute

B) Both inputs and outputs are images

C) Input is an attribute, output is an image

D) It emulates human cognition

Correct Answer: B) Both inputs and outputs are images

Explanation (EN): A low-level process is characterized by the fact that both its inputs and outputs are images (e.g., noise reduction).

الشرح (AR): تتميز العمليات منخفضة المستوى بأن مدخلاتها ومخرجاتها عبارة عن صور (مثل تقليل الضوضاء).

________________________________________

## Q5: What characterizes a mid-level image processing operation?

A) Its inputs and outputs are both images

B) It performs cognitive functions

C) Its inputs are images, but its outputs are attributes extracted from them

D) It only compresses the image

Correct Answer: C) Its inputs are images, but its outputs are attributes extracted from them

Explanation (EN): Mid-level processing involves tasks like segmentation and classification, outputting attributes like edges or contours.

الشرح (AR): تتضمن المعالجة متوسطة المستوى استخراج سمات من الصورة مثل الحواف والحدود.

________________________________________

## Q6: One of the earliest applications of digital images was in the newspaper industry using the Bartlane system. What did this system accomplish?

A) Reduced picture transmission time across the Atlantic from over a week to less than three hours

B) Took the first picture of the moon

C) Discovered X-rays

D) Invented the CAT scan

Correct Answer: A) Reduced picture transmission time across the Atlantic from over a week to less than three hours

Explanation (EN): Introduced in the early 1920s, the Bartlane system sent pictures by submarine cable, reducing transmission time significantly.

الشرح (AR): قلل نظام بارتلين في العشرينيات من زمن نقل الصور عبر المحيط الأطلسي من أسبوع إلى أقل من ثلاث ساعات.

________________________________________

## Q7: The early Bartlane systems in the 1920s were capable of coding images in how many distinct levels of gray?

A) 2

B) 5

C) 15

D) 256

Correct Answer: B) 5

Explanation (EN): The early Bartlane systems coded images in five distinct levels of gray.

الشرح (AR): كانت أنظمة بارتلين المبكرة قادرة على تشفير الصور في 5 مستويات مختلفة من الرمادي.

________________________________________

## Q8: By 1929, the Bartlane system's capability was increased to how many levels of gray?

A) 5

B) 10

C) 15

D) 50

Correct Answer: C) 15

Explanation (EN): The capability of the Bartlane system was increased to 15 levels of gray in 1929.

الشرح (AR): بحلول عام 1929، زادت قدرة نظام بارتلين إلى 15 مستوى من الرمادي.

________________________________________

## Q9: Who introduced the two key concepts of a memory to hold a stored program and conditional branching?

A) John von Neumann

B) Wilhelm Conrad Roentgen

C) Allan M. Cormack

D) Godfrey N. Hounsfield

Correct Answer: A) John von Neumann

Explanation (EN): John von Neumann introduced these concepts in the 1940s, which became the foundation of the central processing unit (CPU).

الشرح (AR): جون فون نيومان هو من قدم مفاهيم الذاكرة لتخزين البرامج والتفرع المشروط في الأربعينيات.

________________________________________

## Q10: In what year was the transistor invented at Bell Laboratories?

A) 1920

B) 1948

C) 1958

D) 1970

Correct Answer: B) 1948

Explanation (EN): The transistor was invented at Bell Laboratories in 1948, a key advance in computing.

الشرح (AR): تم اختراع الترانزستور في مختبرات بيل عام 1948، مما شكل تقدمًا رئيسيًا في الحوسبة.

________________________________________

## Q11: Which spacecraft took the first image of the moon processed by a computer to correct image distortion in 1964?

A) Apollo 11

B) Surveyor 1

C) Mariner 4

D) Ranger 7

Correct Answer: D) Ranger 7

Explanation (EN): Ranger 7 took the first picture of the moon by a U.S. spacecraft in 1964, and the images were processed to correct distortion.

الشرح (AR): المركبة رينجر 7 التقطت أول صورة للقمر تمت معالجتها بواسطة الحاسوب لتصحيح التشوه في عام 1964.

________________________________________

## Q12: Who invented Computerized Axial Tomography (CAT)?

A) John von Neumann

B) Wilhelm Conrad Roentgen

C) Sir Godfrey N. Hounsfield and Professor Allan M. Cormack

D) Richard E. Woods

Correct Answer: C) Sir Godfrey N. Hounsfield and Professor Allan M. Cormack

Explanation (EN): CAT was invented independently by Hounsfield and Cormack, who shared the 1979 Nobel Prize.

الشرح (AR): اخترع التصوير المقطعي (CAT) كل من هاونسفيلد وكورماك وحصلا على جائزة نوبل في عام 1979.

________________________________________

## Q13: Who discovered X-rays in 1895?

A) John von Neumann

B) Wilhelm Conrad Roentgen

C) Sir Godfrey N. Hounsfield

D) Allan M. Cormack

Correct Answer: B) Wilhelm Conrad Roentgen

Explanation (EN): X-rays were discovered in 1895 by Wilhelm Conrad Roentgen.

الشرح (AR): اكتشف فيلهلم كونراد رونتجن الأشعة السينية في عام 1895.

________________________________________

## Q14: Which of the following is a major use of gamma-ray imaging?

A) Radar

B) Positron Emission Tomography (PET)

C) Magnetic Resonance Imaging (MRI)

D) Ultrasound

Correct Answer: B) Positron Emission Tomography (PET)

Explanation (EN): PET is a major modality of nuclear imaging that relies on gamma-ray emission.

الشرح (AR): التصوير المقطعي بالإصدار البوزيتروني (PET) هو أحد الاستخدامات الرئيسية لتصوير أشعة غاما.

________________________________________

## Q15: In PET imaging, what causes the emission of gamma rays?

A) Electrons striking an anode

B) Magnetic fields aligning protons

C) Annihilation of a positron and an electron

D) High-frequency sound waves

Correct Answer: C) Annihilation of a positron and an electron

Explanation (EN): When a positron meets an electron, both are annihilated and two gamma rays are given off.

الشرح (AR): عندما يلتقي بوزيترون بإلكترون، يفني كل منهما الآخر وينبعث شعاعا غاما.

________________________________________

## Q16: How are X-rays typically generated for medical and industrial imaging?

A) By heating a radioactive isotope

B) Using an X-ray tube where electrons strike a positively charged anode

C) By emitting sound pulses

D) By capturing natural gamma radiation

Correct Answer: B) Using an X-ray tube where electrons strike a positively charged anode

Explanation (EN): X-rays are generated in a vacuum tube where heated electrons flow at high speed and strike a positively charged anode.

الشرح (AR): تتولد الأشعة السينية في أنبوب تفريغ حيث تصطدم الإلكترونات السريعة بأنود موجب الشحنة.

________________________________________

## Q17: In contrast enhancement radiography, what is the procedure used to obtain images of blood vessels called?

A) Angiography

B) Lithography

C) Tomography

D) Ultrasound

Correct Answer: A) Angiography

Explanation (EN): Angiography uses a catheter and a contrast medium to enhance the contrast of blood vessels.

الشرح (AR): تصوير الأوعية الدموية (Angiography) يستخدم قسطرة ومادة تباين للحصول على صور واضحة للأوعية.

________________________________________

## Q18: Which imaging modality is commonly used for inspecting electronic circuit boards for manufacturing flaws?

A) Gamma-ray imaging

B) Ultrasound

C) X-ray imaging

D) MRI

Correct Answer: C) X-ray imaging

Explanation (EN): X-rays are heavily used in industrial processes, such as inspecting electronic circuit boards for missing components.

الشرح (AR): تُستخدم الأشعة السينية بشكل شائع في الصناعة، مثل فحص لوحات الدوائر الإلكترونية.

________________________________________

## Q19: Which band of the electromagnetic spectrum is used in fluorescence microscopy?

A) Radio waves

B) Ultraviolet

C) Microwaves

D) Infrared

Correct Answer: B) Ultraviolet

Explanation (EN): Ultraviolet light is used in fluorescence microscopy to excite electrons, which then emit visible light.

الشرح (AR): تُستخدم الأشعة فوق البنفسجية في الفحص المجهري الفلوري لإثارة الإلكترونات التي تشع لاحقاً ضوءاً مرئياً.

________________________________________

## Q20: In NASA's LANDSAT satellites, Band 4 corresponds to which part of the spectrum?

A) Visible blue

B) Thermal infrared

C) Near infrared

D) Visible green

Correct Answer: C) Near infrared

Explanation (EN): According to Table 1.1, Band 4 (0.78-0.90 µm) is the Near infrared band, used for biomass and shoreline mapping.

الشرح (AR): النطاق 4 في أقمار لاندسات يتوافق مع الأشعة تحت الحمراء القريبة ويستخدم لرسم خرائط الكتلة الحيوية.

________________________________________

## Q21: Which band is primarily used to measure plant vigor in LANDSAT satellites?

A) Band 1 (Visible blue)

B) Band 2 (Visible green)

C) Band 6 (Thermal infrared)

D) Band 7 (Short-wave infrared)

Correct Answer: B) Band 2 (Visible green)

Explanation (EN): Table 1.1 indicates that Band 2 (Visible green) is used to measure plant vigor.

الشرح (AR): يُستخدم النطاق 2 (الأخضر المرئي) بشكل أساسي لقياس حيوية النباتات.

________________________________________

## Q22: The 'Nighttime Lights of the World' dataset uses which imaging band to observe human settlements?

A) Ultraviolet

B) Microwave

C) Infrared

D) X-ray

Correct Answer: C) Infrared

Explanation (EN): Infrared imaging (10.0 to 13.4 µm) is used by NOAA/DMSP satellites to observe visible/near infrared emissions from cities.

الشرح (AR): يُستخدم التصوير بالأشعة تحت الحمراء لمراقبة المستوطنات البشرية واستخدام الطاقة ليلاً.

________________________________________

## Q23: What is the principal application of imaging in the microwave band?

A) Lithography

B) Radar

C) MRI

D) Fluorescence microscopy

Correct Answer: B) Radar

Explanation (EN): The principal application in the microwave band is radar, which can collect data regardless of weather.

الشرح (AR): التطبيق الرئيسي في نطاق الميكروويف هو الرادار، والذي يمكنه جمع البيانات بغض النظر عن الطقس.

________________________________________

## Q24: Which of the following is a unique capability of imaging radar?

A) It uses a stream of electrons

B) It requires a radioactive isotope

C) It can penetrate clouds and gather data regardless of weather

D) It records natural gamma radiation

Correct Answer: C) It can penetrate clouds and gather data regardless of weather

Explanation (EN): Imaging radar provides its own illumination and can see through clouds, vegetation, and ice.

الشرح (AR): الرادار يوفر إضاءته الخاصة (نبضات الميكروويف) ويمكنه اختراق السحب والتقاط البيانات في أي طقس.

________________________________________

## Q25: Magnetic Resonance Imaging (MRI) operates in which band of the electromagnetic spectrum?

A) X-ray

B) Ultraviolet

C) Radio wave

D) Gamma ray

Correct Answer: C) Radio wave

Explanation (EN): MRI places a patient in a powerful magnet and uses radio waves (radio band) to generate images.

الشرح (AR): يعمل التصوير بالرنين المغناطيسي (MRI) باستخدام موجات الراديو ضمن مجال مغناطيسي قوي.

________________________________________

## Q26: In marine geological exploration, what is typically used as the energy source for acoustic imaging?

A) A large flat steel plate vibrated by a truck

B) Two air guns towed behind a ship

C) An X-ray tube

D) A cathode ray tube

Correct Answer: B) Two air guns towed behind a ship

Explanation (EN): For marine acoustic imaging, two air guns are typically towed behind a ship to generate sound waves.

الشرح (AR): في الاستكشاف البحري الجيولوجي، تُستخدم مسدسات الهواء المقطورة خلف السفينة لتوليد الموجات الصوتية.

________________________________________

## Q27: What frequency range is typically used for medical ultrasound imaging?

A) 10 to 50 Hz

B) 1 to 5 kHz

C) 1 to 5 MHz

D) 1 to 5 GHz

Correct Answer: C) 1 to 5 MHz

Explanation (EN): Medical ultrasound systems transmit high-frequency sound pulses in the range of 1 to 5 MHz.

الشرح (AR): تستخدم أنظمة الموجات فوق الصوتية الطبية نبضات صوتية عالية التردد تتراوح بين 1 إلى 5 ميجا هرتز.

________________________________________

## Q28: How does a Transmission Electron Microscope (TEM) form an image?

A) It records the returning sound waves

B) It shines a beam of electrons through a thin specimen

C) It scans a beam across the surface to produce light on a phosphor screen

D) It detects natural gamma radiation

Correct Answer: B) It shines a beam of electrons through a thin specimen

Explanation (EN): A TEM transmits a beam of electrons through a specimen, much like a slide projector.

الشرح (AR): يُمرر المجهر الإلكتروني النافذ (TEM) شعاعًا من الإلكترونات عبر عينة رقيقة لتكوين الصورة.

________________________________________

## Q29: What is the principal difference between TEM and SEM?

A) TEM uses sound; SEM uses light

B) TEM uses X-rays; SEM uses electrons

C) TEM requires thin samples and transmits electrons; SEM is for bulky samples and scans the surface

D) There is no difference

Correct Answer: C) TEM requires thin samples and transmits electrons; SEM is for bulky samples and scans the surface

Explanation (EN): TEM shines electrons through a thin sample, whereas SEM scans the electron beam across the surface of a bulky sample.

الشرح (AR): يعتمد TEM على تمرير الإلكترونات عبر عينة رقيقة، بينما يقوم SEM بمسح سطح العينات السميكة.

________________________________________

## Q30: What is a fractal in the context of computer-generated images?

A) An image obtained from an electron microscope

B) An iterative reproduction of a basic pattern according to mathematical rules

C) A digitized X-ray image

D) A type of radar output

Correct Answer: B) An iterative reproduction of a basic pattern according to mathematical rules

Explanation (EN): A fractal is a computer-generated image created by iterativly reproducing a pattern based on mathematical rules.

الشرح (AR): الفركتل هو صورة يتم توليدها حاسوبيًا من خلال تكرار نمط أساسي وفقًا لقواعد رياضية.

________________________________________

## Q31: Which fundamental step of digital image processing involves manipulating an image so the result is subjectively more suitable for a specific application?

A) Image restoration

B) Image enhancement

C) Segmentation

D) Compression

Correct Answer: B) Image enhancement

Explanation (EN): Image enhancement is subjective and deals with manipulating an image to improve its appearance for a specific task.

الشرح (AR): تحسين الصورة هو عملية ذاتية (Subjective) تهدف لمعالجة الصورة لتكون أكثر ملاءمة لتطبيق معين.

________________________________________

## Q32: How does image restoration differ from image enhancement?

A) Restoration is subjective; enhancement is objective

B) Restoration is objective and based on mathematical/probabilistic models

C) Restoration only applies to color images

D) There is no difference

Correct Answer: B) Restoration is objective and based on mathematical/probabilistic models

Explanation (EN): Unlike subjective enhancement, restoration is objective and relies on mathematical models of image degradation.

الشرح (AR): على عكس التحسين الذاتي، تعتبر استعادة الصورة عملية موضوعية تستند إلى نماذج رياضية لتشوه الصورة.

________________________________________

## Q33: Which process deals with tools for extracting image components that are useful in the representation and description of shape?

A) Color image processing

B) Compression

C) Morphological processing

D) Wavelets

Correct Answer: C) Morphological processing

Explanation (EN): Morphological processing involves extracting image components useful for shape representation.

الشرح (AR): تتعامل المعالجة المورفولوجية مع استخراج مكونات الصورة المفيدة في تمثيل ووصف الشكل.

________________________________________

## Q34: Why is segmentation considered one of the most difficult tasks in digital image processing?

A) It requires massive storage

B) Rugged autonomous segmentation is difficult but crucial for recognizing individual objects

C) It only works on 3-D images

D) It requires human intervention for every pixel

Correct Answer: B) Rugged autonomous segmentation is difficult but crucial for recognizing individual objects

Explanation (EN): Autonomous segmentation (partitioning an image into objects) is extremely difficult, and its failure guarantees the failure of subsequent recognition.

الشرح (AR): تجزئة الصورة (Segmentation) تلقائياً أمر بالغ الصعوبة، وفشله يؤدي إلى فشل التعرف الآلي على الأجسام.

________________________________________

## Q35: Feature extraction consists of which two components?

A) Enhancement and restoration

B) Acquisition and display

C) Feature detection and feature description

D) Segmentation and clustering

Correct Answer: C) Feature detection and feature description

Explanation (EN): Feature extraction includes finding features (detection) and assigning quantitative attributes to them (description).

الشرح (AR): تتكون استخراج الميزات من خطوتين: اكتشاف الميزات (Detection) ووصف الميزات (Description) كمياً.

________________________________________

## Q36: What is the purpose of image pattern classification?

A) To reduce storage space

B) To partition the image into objects

C) To assign a label to an object based on its feature descriptors

D) To increase the resolution of the image

Correct Answer: C) To assign a label to an object based on its feature descriptors

Explanation (EN): Classification assigns a specific label (like 'vehicle') to an object based on the features extracted.

الشرح (AR): التصنيف هو العملية التي تمنح ملصقًا (مثل 'مركبة') لجسم ما بناءً على واصفات ميزاته.

________________________________________

## Q37: In an image processing system, what is the function of the digitizer?

A) To display the final image

B) To convert the electrical output of physical sensors into digital form

C) To compress the image

D) To store the image permanently

Correct Answer: B) To convert the electrical output of physical sensors into digital form

Explanation (EN): A digitizer converts the analog output of the physical sensor into digital data for the computer.

الشرح (AR): يقوم جهاز الرقمنة (Digitizer) بتحويل الخرج الكهربائي للمستشعرات المادية إلى بيانات رقمية.

________________________________________

## Q38: What is the primary characteristic of an Arithmetic Logic Unit (ALU) used as front-end specialized hardware in image processing?

A) It stores images for years

B) It performs arithmetic and logical operations in parallel on entire images at high speed

C) It prints the images

D) It captures the sound waves

Correct Answer: B) It performs arithmetic and logical operations in parallel on entire images at high speed

Explanation (EN): An ALU processes entire images in parallel for fast operations, such as averaging video images at 30 frames/s.

الشرح (AR): وحدة الحساب والمنطق في العتاد المتخصص تنفذ عمليات متوازية على الصور بأكملها بسرعة عالية جداً.

________________________________________

## Q39: Which type of mass storage is characterized by frequent access and usually consists of magnetic disks?

A) Short-term storage

B) On-line storage

C) Archival storage

D) Frame buffers

Correct Answer: B) On-line storage

Explanation (EN): On-line storage implies fast, frequent access to stored data, typically using magnetic or optical disks.

الشرح (AR): يتميز التخزين المتصل (On-line storage) بكثرة وسرعة الوصول للبيانات ويستخدم عادة الأقراص المغناطيسية.

________________________________________

## Q40: Frame buffers are typically associated with which type of storage?

A) Archival storage

B) On-line storage

C) Short-term storage

D) Cloud storage

Correct Answer: C) Short-term storage

Explanation (EN): Frame buffers provide short-term storage for rapid access, allowing instantaneous zoom, pan, and scroll at video rates.

الشرح (AR): تُستخدموا مخازن الإطارات (Frame buffers) كتخزين قصير الأجل للوصول السريع للصور أثناء المعالجة.

________________________________________

## Q41: What is the key consideration in image transmission across networks due to the large amount of data involved?

A) Bandwidth

B) Monitor size

C) Digitizer type

D) Sensor resolution

Correct Answer: A) Bandwidth

Explanation (EN): Because image processing involves massive amounts of data, transmission bandwidth is the key consideration in networking.

الشرح (AR): نظرًا للكم الهائل من البيانات في معالجة الصور، يعتبر عرض النطاق الترددي (Bandwidth) العامل الأهم عند النقل عبر الشبكات.

________________________________________

## Q42: Which operation is represented by the double-headed arrows connecting the 'Knowledge base' to the processing modules in the fundamental steps diagram?

A) Permanent storage

B) Display rendering

C) Guiding operation and controlling interaction between modules

D) Image printing

Correct Answer: C) Guiding operation and controlling interaction between modules

Explanation (EN): The double-headed arrows indicate that the knowledge base guides the operation of modules and controls interactions between them.

الشرح (AR): الأسهم المزدوجة تشير إلى أن قاعدة المعرفة توجه تشغيل الوحدات وتتحكم في التفاعل بينها.

________________________________________