# Image Segmentation MCQs (Chapter 10)

## Q1: Image segmentation algorithms are generally based on one of two basic properties of intensity values. What are they?

A) Discontinuity and similarity

B) Brightness and contrast

C) Frequency and spatial domain

D) Smoothing and sharpening

Correct Answer: A

Explanation: Segmentation is based on abrupt changes in intensity (discontinuity) or partitioning based on criteria (similarity).

الشرح (عربي): يعتمد التقسيم على التغيرات المفاجئة في شدة الإضاءة (الانقطاع) أو تقسيم الصورة بناءً على معايير محددة (التشابه).

________________________________________

## Q2: Which of the following is based on the discontinuity property?

A) Region growing

B) Thresholding

C) Edge detection

D) Region merging

Correct Answer: C

Explanation: Edge detection relies on finding abrupt changes in image intensity, which represents discontinuities.

الشرح (عربي): يعتمد اكتشاف الحواف على إيجاد التغيرات المفاجئة في شدة إضاءة الصورة، وهو ما يمثل الانقطاعات.

________________________________________

## Q3: Which segmentation technique is an example of similarity-based partitioning?

A) Point detection

B) Edge detection

C) Thresholding

D) Line detection

Correct Answer: C

Explanation: Thresholding segments an image into regions of similar intensity, thus relying on similarity.

الشرح (عربي): يقوم العتب بتقسيم الصورة إلى مناطق ذات شدة إضاءة متشابهة، وبالتالي يعتمد على التشابه.

________________________________________

## Q4: For point detection using a standard 3x3 mask, what is the sum of its coefficients?

A) 1

B) 8

C) 0

D) -1

Correct Answer: C

Explanation: Derivative masks used for detecting points and edges sum to zero to give no response in areas of constant intensity.

الشرح (عربي): مجموع معاملات الأقنعة المشتقة المستخدمة لاكتشاف النقاط والحواف يساوي صفراً لعدم إعطاء أي استجابة في مناطق الإضاءة الثابتة.

________________________________________

## Q5: What mathematical operator is typically used for point detection?

A) Gradient

B) Laplacian

C) Sobel

D) Fourier Transform

Correct Answer: B

Explanation: The Laplacian, a second-order derivative, is highly responsive to fine detail like isolated points.

الشرح (عربي): اللابلاسيان، وهو مشتق من الدرجة الثانية، شديد الاستجابة للتفاصيل الدقيقة مثل النقاط المعزولة.

________________________________________

## Q6: A horizontal line mask will produce the strongest response for lines oriented at what angle?

A) 0 degrees

B) 45 degrees

C) 90 degrees

D) 135 degrees

Correct Answer: A

Explanation: A horizontal line mask is designed specifically to detect horizontal lines (0 degrees).

الشرح (عربي): تم تصميم قناع الخط الأفقي خصيصًا لاكتشاف الخطوط الأفقية (0 درجة).

________________________________________

## Q7: The first derivative of an image is typically computed using the magnitude of the:

A) Gradient

B) Laplacian

C) Median

D) Histogram

Correct Answer: A

Explanation: The gradient is a vector whose magnitude gives the maximum rate of change, which represents the first derivative.

الشرح (عربي): التدرج (Gradient) هو متجه يعطي مقداره أقصى معدل للتغير، والذي يمثل المشتقة الأولى.

________________________________________

## Q8: The Sobel operator uses which of the following mask sizes for basic edge detection?

A) 2x2

B) 3x3

C) 4x4

D) 5x5

Correct Answer: B

Explanation: The standard Sobel operators use 3x3 masks to compute horizontal and vertical approximations of the derivative.

الشرح (عربي): تستخدم عوامل سوبل القياسية أقنعة 3x3 لحساب التقريبات الأفقية والعمودية للمشتقة.

________________________________________

## Q9: What advantage does the Sobel operator have over the Prewitt operator?

A) It is faster

B) It uses a 2x2 mask

C) It provides better noise suppression

D) It is a second derivative operator

Correct Answer: C

Explanation: The Sobel operator uses a weight of 2 in the center, providing some smoothing which suppresses noise better than Prewitt.

الشرح (عربي): يستخدم عامل سوبل وزناً قدره 2 في المنتصف، مما يوفر بعض التنعيم الذي يقلل من الضوضاء بشكل أفضل من عامل بريويت.

________________________________________

## Q10: The Roberts cross-gradient operators use masks of what dimension?

A) 1x1

B) 2x2

C) 3x3

D) 4x4

Correct Answer: B

Explanation: The Roberts operators are one of the oldest edge detectors and use simple 2x2 masks.

الشرح (عربي): عوامل روبرتس هي من أقدم مكتشفات الحواف وتستخدم أقنعة بسيطة بأبعاد 2x2.

________________________________________

## Q11: The Laplacian is a:

A) First-order derivative operator

B) Second-order derivative operator

C) Smoothing filter

D) Morphological operator

Correct Answer: B

Explanation: The Laplacian computes the sum of unmixed second derivatives, making it a second-order operator.

الشرح (عربي): يحسب اللابلاسيان مجموع المشتقات الثانية غير المختلطة، مما يجعله عاملاً من الدرجة الثانية.

________________________________________

## Q12: A primary disadvantage of using the Laplacian alone for edge detection is:

A) It produces thick edges

B) It is insensitive to noise

C) It is highly sensitive to noise

D) It only detects vertical lines

Correct Answer: C

Explanation: Because it is a second-order derivative, the Laplacian amplifies high-frequency noise significantly.

الشرح (عربي): نظراً لكونه مشتقاً من الدرجة الثانية، فإن اللابلاسيان يضخم الضوضاء عالية التردد بشكل كبير.

________________________________________

## Q13: The Marr-Hildreth edge detector uses which function?

A) Difference of Gaussians (DoG)

B) Laplacian of a Gaussian (LoG)

C) Canny function

D) Sobel function

Correct Answer: B

Explanation: The Marr-Hildreth algorithm filters the image with a Gaussian before applying the Laplacian, hence LoG.

الشرح (عربي): تقوم خوارزمية مار-هيلدريث بتصفية الصورة باستخدام دالة غاوسية قبل تطبيق اللابلاسيان، ومن هنا جاءت تسمية LoG.

________________________________________

## Q14: In the Marr-Hildreth detector, edges correspond to what features of the LoG output?

A) Maximum values

B) Minimum values

C) Zero-crossings

D) Average values

Correct Answer: C

Explanation: Zero-crossings of the second derivative correspond to the inflection points (peaks) of the first derivative, indicating an edge.

الشرح (عربي): تتوافق تقاطعات الصفر للمشتقة الثانية مع نقاط الانعطاف (القمم) للمشتقة الأولى، مما يشير إلى وجود حافة.

________________________________________

## Q15: The Canny edge detector uses which technique to narrow wide ridges into 1-pixel thin edges?

A) Double thresholding

B) Gaussian smoothing

C) Non-maximum suppression

D) Region growing

Correct Answer: C

Explanation: Non-maximum suppression suppresses pixels that are not local maxima, effectively thinning the edge.

الشرح (عربي): يقوم الكبح غير الأقصى بتثبيط البكسلات التي ليست قيماً قصوى محلية، مما يؤدي إلى ترقيق الحافة.

________________________________________

## Q16: Which edge detector attempts to minimize an error rate, achieve good localization, and have only a single response to an edge?

A) Sobel

B) Roberts

C) Marr-Hildreth

D) Canny

Correct Answer: D

Explanation: These three criteria were explicitly formulated by John Canny to design the optimal edge detector.

الشرح (عربي): تم صياغة هذه المعايير الثلاثة صراحة من قبل جون كاني لتصميم المكتشف الأمثل للحواف.

________________________________________

## Q17: What is the purpose of hysteresis thresholding in the Canny edge detector?

A) To thin the edges

B) To link broken edges

C) To smooth the image

D) To find the gradient angle

Correct Answer: B

Explanation: Hysteresis uses a high threshold to find strong edges and a low threshold to connect weak edges attached to strong ones.

الشرح (عربي): يستخدم التخلف عتبة عالية للعثور على حواف قوية وعتبة منخفضة لربط الحواف الضعيفة المتصلة بها.

________________________________________

## Q18: Which transform is widely used for linking edge pixels into global parametric shapes such as lines?

A) Fourier transform

B) Wavelet transform

C) Hough transform

D) Radon transform

Correct Answer: C

Explanation: The Hough transform groups edge pixels into object candidates by mapping them to a parameter space.

الشرح (عربي): يجمع تحويل هوف بكسلات الحافة في كائنات مرشحة عن طريق تعيينها إلى فضاء المعلمات.

________________________________________

## Q19: In the normal representation of a line (rho = x*cos(theta) + y*sin(theta)) used in the Hough transform, a single point in the xy-plane maps to what in the rho-theta parameter space?

A) A point

B) A line

C) A sinusoidal curve

D) A circle

Correct Answer: C

Explanation: A point in image space maps to a sinusoid in the rho-theta parameter space.

الشرح (عربي): يتم تعيين نقطة في مساحة الصورة إلى منحنى جيبي في فضاء المعلمات (rho-theta).

________________________________________

## Q20: Thresholding segments an image into objects and background based on what property?

A) Edge direction

B) Intensity values

C) Texture variations

D) Spatial frequency

Correct Answer: B

Explanation: Thresholding compares intensity values against a threshold to separate objects from the background.

الشرح (عربي): تقارن عملية العتب قيم شدة الإضاءة بحد العتبة لفصل الكائنات عن الخلفية.

________________________________________

## Q21: If a threshold is constant for the entire image, it is called:

A) Local thresholding

B) Global thresholding

C) Adaptive thresholding

D) Regional thresholding

Correct Answer: B

Explanation: Global thresholding uses a single threshold value for all pixels in an image.

الشرح (عربي): يستخدم العتب الشامل قيمة عتبة واحدة لجميع البكسلات في الصورة.

________________________________________

## Q22: If the threshold value T depends on the spatial coordinates (x,y), the thresholding method is called:

A) Global

B) Dynamic or variable

C) Optimal

D) Fixed

Correct Answer: B

Explanation: Variable (or adaptive) thresholding changes the threshold across the image based on local spatial properties.

الشرح (عربي): يغير العتب المتغير حد العتبة عبر الصورة بناءً على الخصائص المكانية المحلية.

________________________________________

## Q23: Illumination variations across an image often cause global thresholding to fail. Which technique is most effective in solving this?

A) Increasing the global threshold

B) Variable thresholding

C) Adding Gaussian noise

D) Shrinking the image

Correct Answer: B

Explanation: Variable thresholding compensates for non-uniform illumination by computing thresholds locally.

الشرح (عربي): يعوض العتب المتغير عن الإضاءة غير المنتظمة عن طريق حساب العتبات محلياً.

________________________________________

## Q24: Otsu's method is used for which purpose?

A) Finding the best edge directions

B) Smoothing an image automatically

C) Automatically finding an optimal global threshold

D) Region splitting

Correct Answer: C

Explanation: Otsu's algorithm automatically determines the optimal threshold by maximizing the between-class variance.

الشرح (عربي): تحدد خوارزمية أوتسو تلقائياً العتبة المثلى عن طريق تعظيم التباين بين الفئات.

________________________________________

## Q25: What does Otsu's method aim to maximize?

A) Within-class variance

B) Between-class variance

C) Total variance

D) Mean square error

Correct Answer: B

Explanation: By maximizing the between-class variance, Otsu's method ensures maximum separation between object and background classes.

الشرح (عربي): من خلال تعظيم التباين بين الفئات، تضمن طريقة أوتسو أقصى فصل بين فئات الكائن والخلفية.

________________________________________

## Q26: Which basic assumption does Otsu's method make about the image histogram?

A) It is uniform

B) It contains three equal classes

C) It is bimodal (contains two principal classes)

D) It is perfectly symmetric

Correct Answer: C

Explanation: Otsu's method assumes the histogram has two main peaks representing the background and the foreground.

الشرح (عربي): تفترض طريقة أوتسو أن المدرج التكراري يحتوي على قمتين رئيسيتين تمثلان الخلفية والمقدمة.

________________________________________

## Q27: Region growing is an image segmentation technique that is based on:

A) Discontinuity

B) Similarity

C) Filtering

D) Edge linking

Correct Answer: B

Explanation: Region growing groups pixels based on predefined criteria of similarity, such as intensity or color.

الشرح (عربي): يقوم نمو المنطقة بتجميع البكسلات بناءً على معايير محددة مسبقاً للتشابه، مثل شدة الإضاءة أو اللون.

________________________________________

## Q28: Region growing requires initial pixels to start the growing process. What are these pixels called?

A) Leaves

B) Branches

C) Seeds

D) Roots

Correct Answer: C

Explanation: The initial starting points for the region growing process are known as "seed" points.

الشرح (عربي): تُعرف نقاط البداية الأولية لعملية نمو المنطقة باسم نقاط "البذور".

________________________________________

## Q29: A common issue in region growing where the segmented regions grow beyond their true boundaries is called:

A) Leakage

B) Splitting

C) Over-segmentation

D) Merging

Correct Answer: A

Explanation: Leakage occurs if the connectivity or similarity criteria are too relaxed, causing the region to spill into surrounding areas.

الشرح (عربي): يحدث التسرب إذا كانت معايير الاتصال أو التشابه متساهلة للغاية، مما يتسبب في امتداد المنطقة إلى المناطق المحيطة.

________________________________________

## Q30: The region splitting and merging algorithm is typically implemented using what kind of data structure?

A) Binary tree

B) Linked list

C) Quadtree

D) Hash table

Correct Answer: C

Explanation: A quadtree divides a square region into four smaller quadrants recursively if the region is not homogeneous.

الشرح (عربي): تقسم الشجرة الرباعية (Quadtree) المنطقة المربعة إلى أربعة أرباع أصغر بشكل متكرر إذا كانت المنطقة غير متجانسة.

________________________________________

## Q31: In region splitting and merging, a region is split into four sub-regions if:

A) It satisfies a homogeneity predicate

B) It does not satisfy a homogeneity predicate

C) It contains an edge

D) It is completely black

Correct Answer: B

Explanation: If a region is not homogeneous according to a predefined predicate, it must be split into smaller subregions.

الشرح (عربي): إذا لم تكن المنطقة متجانسة وفقاً لمسند محدد مسبقاً، فيجب تقسيمها إلى مناطق فرعية أصغر.

________________________________________

## Q32: In the merging step of the split-and-merge algorithm, two adjacent regions are merged if:

A) Their combined area is larger than a threshold

B) Their combined region satisfies the homogeneity predicate

C) They have different intensity means

D) They share exactly one pixel

Correct Answer: B

Explanation: Adjacent regions that satisfy the similarity condition when combined are merged back together.

الشرح (عربي): يتم دمج المناطق المتجاورة التي تلبي شرط التشابه عند دمجها معاً مرة أخرى.

________________________________________

## Q33: Morphological watershed segmentation visualizes an image as:

A) A set of discrete particles

B) A one-dimensional frequency signal

C) A 3D topographic surface

D) A color wheel

Correct Answer: C

Explanation: Watershed transforms view image intensities as elevations, treating the image as a topographic relief.

الشرح (عربي): تعرض تحويلات مستجمعات المياه شدة إضاءة الصورة كارتفاعات، وتتعامل مع الصورة كتضاريس طوبوغرافية.

________________________________________

## Q34: Direct application of the watershed transform to a gradient image usually leads to what problem?

A) Under-segmentation

B) Over-segmentation

C) Image blurring

D) Edge disappearance

Correct Answer: B

Explanation: Noise and local gradient irregularities create many local minima, leading to severe over-segmentation.

الشرح (عربي): تخلق الضوضاء والمخالفات المحلية في التدرج العديد من الحدود الدنيا المحلية، مما يؤدي إلى تجزئة مفرطة.

________________________________________

## Q35: To mitigate the over-segmentation problem in watersheds, what approach is commonly used?

A) Low-pass filtering exclusively

B) Edge suppression

C) Marker-controlled watershed segmentation

D) Global thresholding

Correct Answer: C

Explanation: Markers denote the specific locations of regions, restricting the valid local minima to prevent over-segmentation.

الشرح (عربي): تشير العلامات (Markers) إلى المواقع المحددة للمناطق، وتقيد الحدود الدنيا المحلية الصالحة لمنع التجزئة المفرطة.

________________________________________

## Q36: What is a "superpixel" in the context of image segmentation?

A) A very bright pixel

B) A pixel that contains all color channels

C) A cohesive, localized grouping of pixels that preserve image structure

D) A pixel belonging to an edge

Correct Answer: C

Explanation: Superpixels group pixels into perceptually meaningful atomic regions, reducing the number of primitives for further processing.

الشرح (عربي): تجمع وحدات البكسل الفائقة وحدات البكسل في مناطق ذرية ذات معنى إدراكي، مما يقلل من عدد العناصر الأولية للمعالجة الإضافية.

________________________________________

## 37: One common algorithm for computing superpixels efficiently is:

A) Canny

B) SLIC (Simple Linear Iterative Clustering)

C) Otsu

D) Hough

Correct Answer: B

Explanation: SLIC is a widely used algorithm that adapts k-means clustering to generate superpixels efficiently.

الشرح (عربي): خوارزمية SLIC تكيف تجميع k-means لتوليد وحدات بكسل فائقة بكفاءة.

________________________________________

## Q38: Graph cuts segmentation models the segmentation problem as finding:

A) The longest path in a graph

B) The minimum cut or maximum flow

C) The maximum spanning tree

D) The shortest path between all nodes

Correct Answer: B

Explanation: Graph cuts find a partition of the graph that minimizes the cut capacity, separating the object from the background.

الشرح (عربي): تجد مقاطع الرسم البياني تقسيماً للرسم البياني يقلل من سعة القطع، ويفصل الكائن عن الخلفية.

________________________________________

## Q39: In graph cuts, nodes typically correspond to what image elements?

A) Image boundaries

B) Threshold values

C) Pixels or regions

D) Morphological structuring elements

Correct Answer: C

Explanation: Each pixel (or superpixel) is represented as a node in the graph.

الشرح (عربي): يتم تمثيل كل بكسل كعقدة في الرسم البياني.

________________________________________

## Q40: In terms of computational approach, graph-based segmentation is primarily a form of:

A) Energy minimization

B) High-pass filtering

C) Histogram equalization

D) Geometric transformation

Correct Answer: A

Explanation: The method constructs an energy function that models region similarity and boundary costs, which is then minimized.

الشرح (عربي): تبني الطريقة دالة طاقة تصيغ تشابه المنطقة وتكاليف الحدود، والتي يتم تقليلها بعد ذلك.

________________________________________