# MCQs - Data Communications (Ch1 & Ch2)

1. Four fundamental characteristics of a data communications system effectiveness are delivery, accuracy, timeliness, and...

A) Security

B) Jitter

C) Protocol

D) Performance

Correct Answer: B

Explanation (EN): The effectiveness depends on delivery, accuracy, timeliness, and jitter.

الشرح (عربي): تعتمد الفعالية على التسليم، والدقة، والتوقيت، والتشوه الزمني (Jitter).

------------------------------------------------------------

2. Which component is the physical path by which a message travels?

A) Sender

B) Protocol

C) Transmission medium

D) Receiver

Correct Answer: C

Explanation (EN): The transmission medium is the physical path by which a message travels from sender to receiver.

الشرح (عربي): وسيط النقل هو المسار المادي الذي تنتقل عبره الرسالة من المرسل إلى المستقبل.

------------------------------------------------------------

3. Which data flow mode allows both stations to transmit and receive simultaneously?

A) Simplex

B) Half-duplex

C) Full-duplex

D) Auto-duplex

Correct Answer: C

Explanation (EN): In full-duplex mode, both stations can transmit and receive simultaneously.

الشرح (عربي): في وضع الاتصال المزدوج الكامل (Full-duplex)، يمكن لكلتا المحطتين الإرسال والاستقبال في نفس الوقت.

------------------------------------------------------------

4. How many I/O ports are required for each device in a fully connected mesh topology with n nodes?

A) n

B) n-1

C) n+1

D) n(n-1)

Correct Answer: B

Explanation (EN): Every device on the network must have n-1 input/output (I/O) ports to connect to the other n-1 stations.

الشرح (عربي): يجب أن يحتوي كل جهاز على n-1 من منافذ الإدخال/الإخراج للاتصال بالمحطات الأخرى.

------------------------------------------------------------

5. In which topology does each device have a dedicated point-to-point link only to a central controller (hub)?

A) Mesh

B) Bus

C) Ring

D) Star

Correct Answer: D

Explanation (EN): In a star topology, each device has a dedicated point-to-point link only to a central controller, usually called a hub.

الشرح (عربي): في الهيكل النجمي (Star topology)، يمتلك كل جهاز رابطًا مخصصًا فقط مع وحدة تحكم مركزية (Hub).

------------------------------------------------------------

6. Which topology features a multipoint connection where one long cable acts as a backbone to link all devices?

A) Bus

B) Star

C) Ring

D) Mesh

Correct Answer: A

Explanation (EN): A bus topology is multipoint. One long cable acts as a backbone to link all the devices in a network.

الشرح (عربي): هيكل الناقل (Bus topology) هو اتصال متعدد النقاط، حيث يعمل كابل طويل كعمود فقري لربط جميع الأجهزة.

------------------------------------------------------------

7. What is an internetwork (or internet)?

A) A single local area network

B) Two or more networks connected together

C) A point-to-point WAN

D) A single host computer

Correct Answer: B

Explanation (EN): When two or more networks are connected, they make an internetwork, or internet.

الشرح (عربي): عندما يتم توصيل شبكتين أو أكثر، فإنها تشكل شبكة متداخلة أو إنترنت.

------------------------------------------------------------

8. The TCP/IP protocol suite is composed of how many layers?

A) 4

B) 5

C) 6

D) 7

Correct Answer: B

Explanation (EN): The TCP/IP protocol suite is defined as five layers: physical, data-link, network, transport, and application.

الشرح (عربي): تتكون حزمة بروتوكولات TCP/IP من خمس طبقات: المادية، وربط البيانات، والشبكة، والنقل، والتطبيق.

------------------------------------------------------------

9. Which TCP/IP layer is responsible for carrying individual bits in a frame across the link?

A) Data-link

B) Network

C) Physical

D) Transport

Correct Answer: C

Explanation (EN): The physical layer is responsible for carrying individual bits in a frame across the link.

الشرح (عربي): الطبقة المادية (Physical layer) مسؤولة عن حمل البتات الفردية عبر الرابط.

------------------------------------------------------------

10. The network layer in the TCP/IP protocol suite is responsible for:

A) Hop-to-hop delivery

B) Process-to-process delivery

C) Source-to-destination delivery of a packet across multiple networks

D) Providing services to the application

Correct Answer: C

Explanation (EN): The network layer is responsible for creating a connection between the source and destination computers across multiple networks.

الشرح (عربي): طبقة الشبكة (Network layer) مسؤولة عن توصيل الحزمة من المصدر إلى الوجهة عبر شبكات متعددة.

------------------------------------------------------------

11. Which layer provides process-to-process delivery of the entire message?

A) Application

B) Transport

C) Network

D) Data-link

Correct Answer: B

Explanation (EN): The transport layer is responsible for process-to-process delivery of the entire message.

الشرح (عربي): طبقة النقل (Transport layer) مسؤولة عن التوصيل من عملية إلى عملية (process-to-process) للرسالة بأكملها.

------------------------------------------------------------

12. Which two layers of the OSI model are missing from the TCP/IP protocol suite?

A) Session and Presentation

B) Network and Transport

C) Data-link and Physical

D) Application and Session

Correct Answer: A

Explanation (EN): When comparing the two models, the session and presentation layers are missing from the TCP/IP suite.

الشرح (عربي): عند مقارنة النموذجين، نجد أن طبقتي الجلسة (Session) والتقديم (Presentation) مفقودتان من حزمة TCP/IP.

------------------------------------------------------------

13. An analog signal that cannot be decomposed into simpler signals is called a:

A) Composite signal

B) Periodic digital signal

C) Simple periodic analog signal

D) Aperiodic analog signal

Correct Answer: C

Explanation (EN): A simple periodic analog signal, a sine wave, cannot be decomposed into simpler signals.

الشرح (عربي): الإشارة التناظرية الدورية البسيطة، مثل الموجة الجيبية، لا يمكن تفكيكها إلى إشارات أبسط.

------------------------------------------------------------

14. Which parameter of a sine wave is measured in Hertz (Hz)?

A) Peak Amplitude

B) Phase

C) Period

D) Frequency

Correct Answer: D

Explanation (EN): The frequency (f), measured in hertz (Hz), refers to the number of periods in 1 second.

الشرح (عربي): التردد (Frequency) يُقاس بالهرتز (Hz) ويشير إلى عدد الدورات في ثانية واحدة.

------------------------------------------------------------

15. The mathematical relationship between period (T) and frequency (f) is:

A) f = T

B) f = 1/T

C) f = T^2

D) f = c/T

Correct Answer: B

Explanation (EN): Period and frequency are inverses of each other, meaning f = 1/T.

الشرح (عربي): المدة والتردد هما مقلوبان لبعضهما البعض، أي أن التردد يساوي 1 مقسومًا على المدة.

------------------------------------------------------------

16. Which parameter describes the position of the waveform relative to time 0?

A) Frequency

B) Wavelength

C) Phase

D) Amplitude

Correct Answer: C

Explanation (EN): Phase describes the position of the waveform relative to time 0.

الشرح (عربي): الطور (Phase) يصف موضع الموجة بالنسبة للوقت صفر.

------------------------------------------------------------

17. The bandwidth of a composite signal is defined as:

A) The highest frequency in the signal

B) The lowest frequency in the signal

C) The difference between the highest and lowest frequencies

D) The sum of all frequencies

Correct Answer: C

Explanation (EN): The bandwidth is the difference between the lowest and highest frequencies in the signal.

الشرح (عربي): النطاق الترددي (Bandwidth) هو الفرق بين أعلى وأدنى تردد في الإشارة.

------------------------------------------------------------

18. If a digital signal has 4 signal levels, how many bits are sent per level?

A) 1

B) 2

C) 3

D) 4

Correct Answer: B

Explanation (EN): If a signal has L levels, each level needs log2(L) bits. For 4 levels, log2(4) = 2 bits.

الشرح (عربي): إذا كان للإشارة 4 مستويات، فإن كل مستوى يحتاج إلى بتين (2 bits) وفقًا للمعادلة log2(4).

------------------------------------------------------------

19. What is the term used to describe the number of bits sent in 1 second for a digital signal?

A) Bit length

B) Frequency

C) Baud rate

D) Bit rate

Correct Answer: D

Explanation (EN): The bit rate is the number of bits sent in 1 second, expressed in bps.

الشرح (عربي): معدل البت (Bit rate) هو عدد البتات المرسلة في ثانية واحدة، ويقاس بـ bps.

------------------------------------------------------------

20. Sending a digital signal over a channel without changing it to an analog signal is called:

A) Broadband transmission

B) Baseband transmission

C) Modulation

D) Analog transmission

Correct Answer: B

Explanation (EN): Baseband transmission means sending a digital signal over a channel without changing it to an analog signal.

الشرح (عربي): إرسال النطاق الأساسي (Baseband transmission) يعني إرسال إشارة رقمية عبر قناة دون تحويلها إلى إشارة تناظرية.

------------------------------------------------------------

21. Which signal impairment refers to a loss of energy as the signal overcomes the resistance of the medium?

A) Distortion

B) Noise

C) Attenuation

D) Crosstalk

Correct Answer: C

Explanation (EN): Attenuation means a loss of energy when a signal travels through a medium.

الشرح (عربي): التوهين (Attenuation) يعني فقدان الطاقة عندما تنتقل الإشارة عبر وسيط النقل بسبب مقاومته.

------------------------------------------------------------

22. Which unit is used to measure the relative strengths of two signals to calculate attenuation or amplification?

A) Hertz

B) Decibel (dB)

C) Volts

D) Watts

Correct Answer: B

Explanation (EN): The decibel (dB) measures the relative strengths of two signals or one signal at two different points.

الشرح (عربي): الديسيبل (dB) يقيس القوة النسبية لإشارتين أو لإشارة واحدة في نقطتين مختلفتين.

------------------------------------------------------------

23. Distortion in a composite signal occurs primarily because:

A) Electrons have random motion

B) Different frequency components have different propagation speeds

C) The signal loses energy

D) External devices induce signals

Correct Answer: B

Explanation (EN): Distortion occurs because each signal component has its own propagation speed and delay.

الشرح (عربي): يحدث التشوه (Distortion) لأن كل مكون من مكونات الإشارة له سرعة انتشار وتأخير خاص به.

------------------------------------------------------------

24. The random motion of electrons in a wire that creates an extra unwanted signal is called:

A) Impulse noise

B) Crosstalk

C) Induced noise

D) Thermal noise

Correct Answer: D

Explanation (EN): Thermal noise is the random motion of electrons in a wire, creating an extra signal.

الشرح (عربي): الضوضاء الحرارية (Thermal noise) هي الحركة العشوائية للإلكترونات في السلك والتي تخلق إشارة إضافية غير مرغوب فيها.

------------------------------------------------------------

25. The theoretical maximum bit rate of a noiseless channel is defined by:

A) Shannon capacity

B) Nyquist bit rate formula

C) Fourier transform

D) Phase shift formula

Correct Answer: B

Explanation (EN): For a noiseless channel, the Nyquist bit rate formula defines the theoretical maximum bit rate.

الشرح (عربي): بالنسبة لقناة خالية من الضوضاء، تحدد معادلة معدل بت نايكويست الحد الأقصى النظري لمعدل البت.

------------------------------------------------------------

26. Which formula determines the theoretical highest data rate for a noisy channel?

A) Shannon capacity

B) Nyquist bit rate

C) Baud rate formula

D) Attenuation limit

Correct Answer: A

Explanation (EN): Claude Shannon introduced the Shannon capacity formula for a noisy channel.

الشرح (عربي): قدم كلود شانون صيغة سعة شانون (Shannon capacity) لتحديد أعلى معدل بيانات نظري لقناة صاخبة.

------------------------------------------------------------

27. The actual measurement of how fast we can send data through a network is called:

A) Bandwidth in Hertz

B) Potential bandwidth

C) Throughput

D) Frequency range

Correct Answer: C

Explanation (EN): The throughput is an actual measurement of how fast we can send data, while bandwidth is potential.

الشرح (عربي): الإنتاجية (Throughput) هي المقياس الفعلي لمدى سرعة إرسال البيانات، بينما يمثل النطاق الترددي السرعة المحتملة.

------------------------------------------------------------

28. Latency (total delay) consists of propagation delay, transmission delay, queuing delay, and:

A) Processing delay

B) Amplification delay

C) Attenuation delay

D) Refraction delay

Correct Answer: A

Explanation (EN): Latency is the sum of propagation, transmission, queuing, and processing delays.

الشرح (عربي): يتكون التأخير الكلي من تأخير الانتشار، تأخير الإرسال، تأخير الطابور، وتأخير المعالجة (Processing delay).

------------------------------------------------------------

29. The process of converting digital data to digital signals is known as:

A) Line coding

B) Block coding

C) Scrambling

D) Modulation

Correct Answer: A

Explanation (EN): Line coding is the process of converting digital data to digital signals.

الشرح (عربي): تشفير الخط (Line coding) هو عملية تحويل البيانات الرقمية إلى إشارات رقمية.

------------------------------------------------------------

30. The three processes of a Pulse Code Modulation (PCM) encoder are Sampling, Quantizing, and:

A) Multiplexing

B) Encoding

C) Modulating

D) Scrambling

Correct Answer: B

Explanation (EN): A PCM encoder processes are: Sampling, Quantizing, and Encoding.

الشرح (عربي): عمليات تشفير PCM هي: أخذ العينات (Sampling)، التكميم (Quantizing)، والتشفير (Encoding).

------------------------------------------------------------

31. In Delta Modulation (DM), instead of finding the exact amplitude for each sample, it finds:

A) The highest peak amplitude

B) The change from the previous sample

C) The phase shift

D) The frequency deviation

Correct Answer: B

Explanation (EN): DM finds the change from the previous sample rather than the exact amplitude.

الشرح (عربي): في تعديل دلتا (DM)، يتم إيجاد التغيير عن العينة السابقة بدلاً من السعة الدقيقة.

------------------------------------------------------------

32. Which digital-to-analog conversion technique varies the amplitude of the carrier signal to represent data?

A) FSK

B) PSK

C) QAM

D) ASK

Correct Answer: D

Explanation (EN): In Amplitude Shift Keying (ASK), the amplitude of the carrier signal is varied.

الشرح (عربي): في تعديل إزاحة السعة (ASK)، يتم تغيير سعة الإشارة الحاملة لتمثيل البيانات.

------------------------------------------------------------

33. Which modulation mechanism combines changing both the amplitude and phase?

A) FSK

B) QAM

C) ASK

D) PM

Correct Answer: B

Explanation (EN): Quadrature Amplitude Modulation (QAM) combines changing both amplitude and phase.

الشرح (عربي): تعديل سعة التربيع (QAM) يجمع بين تغيير كل من السعة والطور.

------------------------------------------------------------

34. Analog-to-analog conversion (modulation) is necessary when:

A) Only a bandpass channel is available

B) The medium is strictly low-pass

C) The signal is already digital

D) The transmission distance is negligible

Correct Answer: A

Explanation (EN): Modulation is needed if the medium is bandpass in nature or if only a bandpass channel is available.

الشرح (عربي): يعد التعديل ضروريًا إذا كانت القناة المتاحة هي قناة تمرير النطاق (Bandpass channel) فقط.

------------------------------------------------------------

35. In Frequency-Division Multiplexing (FDM), channels can be separated by strips of unused bandwidth called:

A) Carrier bands

B) Guard bands

C) Phase bands

D) Basebands

Correct Answer: B

Explanation (EN): Channels are separated by strips of unused bandwidth—guard bands—to prevent overlapping.

الشرح (عربي): يتم فصل القنوات بشرائط من النطاق الترددي غير المستخدم تسمى نطاقات الحماية (Guard bands) لمنع التداخل.

------------------------------------------------------------

36. Time-Division Multiplexing (TDM) is primarily a:

A) Digital process sharing time

B) Analog process sharing frequency

C) Digital process sharing frequency

D) Analog process sharing time

Correct Answer: A

Explanation (EN): TDM is a digital process that allows several connections to share the bandwidth of a link by sharing time.

الشرح (عربي): TDM هي عملية رقمية تسمح لعدة اتصالات بمشاركة النطاق الترددي عن طريق مشاركة الوقت.

------------------------------------------------------------

37. In a twisted-pair cable, what is the main reason for twisting the two conductors together?

A) To increase physical strength

B) To maintain a balance against noise and crosstalk

C) To reduce the weight of the cable

D) To increase the propagation speed

Correct Answer: B

Explanation (EN): By twisting the pairs, a balance is maintained against interference and crosstalk.

الشرح (عربي): عن طريق لف الأسلاك، يتم الحفاظ على التوازن ضد التداخل (Noise) والتداخل المتبادل (Crosstalk).

------------------------------------------------------------

38. Which guided medium has a central core conductor enclosed in an insulating sheath, encased in an outer metal conductor?

A) Unshielded Twisted Pair

B) Fiber-optic cable

C) Infrared

D) Coaxial cable

Correct Answer: D

Explanation (EN): Coaxial cable has a central core conductor enclosed in an insulating sheath and an outer conductor.

الشرح (عربي): الكابل المحوري (Coaxial cable) يحتوي على موصل مركزي مغلف بغمد عازل، ومحاط بموصل معدني خارجي.

------------------------------------------------------------

39. Optical fibers guide light through a channel using the physical principle of:

A) Refraction into the cladding

B) Reflection off the cladding

C) Thermal radiation

D) Attenuation

Correct Answer: B

Explanation (EN): Optical fibers use reflection to guide light through a channel; light is reflected off the cladding.

الشرح (عربي): تستخدم الألياف الضوئية الانعكاس (Reflection) لتوجيه الضوء؛ حيث ينعكس الضوء عن الغلاف (Cladding).

------------------------------------------------------------

40. Electromagnetic waves ranging in frequencies between 1 and 300 GHz that are unidirectional are called:

A) Radio waves

B) Microwaves

C) Infrared waves

D) Visible light

Correct Answer: B

Explanation (EN): Waves between 1 and 300 GHz are microwaves, which are unidirectional and can be narrowly focused.

الشرح (عربي): الموجات بين 1 و 300 جيجاهرتز هي موجات دقيقة (Microwaves)، وهي أحادية الاتجاه ويمكن تركيزها بدقة.

------------------------------------------------------------