# Data Communications and Networking - MCQs

This document contains 41 Multiple Choice Questions (MCQs) for competitive exam preparation based on the uploaded textbook chapter.

Q1. Which connecting device operates only in the physical layer of the TCP/IP protocol suite?

A) Link-layer switch

B) Router

C) Hub

D) Gateway

Correct Answer: C

Explanation (En): A hub is a device that operates only in the physical layer and broadcasts signals to all ports without filtering.

الشرح (عربي): يعمل الموزع (Hub) كجهاز في الطبقة الفيزيائية فقط ويقوم ببث الإشارات إلى جميع المنافذ دون تصفية.

--------------------------------------------------

Q2. Which device operates in both the physical and the data-link layers?

A) Hub

B) Link-layer switch

C) Repeater

D) Router

Correct Answer: B

Explanation (En): A link-layer switch operates in both the physical and the data-link layers, having filtering capability to forward frames to specific ports.

الشرح (عربي): يعمل محول طبقة الارتباط (Link-layer switch) في كل من الطبقة الفيزيائية وطبقة ارتباط البيانات، ويمتلك قدرة على التصفية لتوجيه الإطارات إلى منافذ محددة.

--------------------------------------------------

Q3. What is the primary difference in functionality between a link-layer switch and a hub?

A) A hub has filtering capability.

B) A link-layer switch has filtering capability.

C) A hub operates in the network layer.

D) A switch broadcasts frames to all ports without checking.

Correct Answer: B

Explanation (En): A link-layer switch has a switching table used for filtering decisions to decide the outgoing port.

الشرح (عربي): يحتوي محول طبقة الارتباط على قدرة التصفية باستخدام جدول لتحديد منفذ الخروج الصحيح، بينما الموزع لا يمتلك هذه القدرة.

--------------------------------------------------

Q4. What type of switch allows stations to be completely unaware of its existence?

A) Transparent switch

B) Static switch

C) Routing switch

D) Hub switch

Correct Answer: A

Explanation (En): A transparent switch is a switch in which the stations are completely unaware of the switch's existence.

الشرح (عربي): المحول الشفاف (Transparent switch) هو المحول الذي لا تدرك المحطات (Stations) وجوده في الشبكة على الإطلاق.

--------------------------------------------------

Q5. How does a transparent switch create its forwarding table automatically?

A) By inspecting the destination addresses

B) By learning from frame movements using source addresses

C) By manual configuration by the administrator

D) By using the Spanning Tree Protocol

Correct Answer: B

Explanation (En): To make a table dynamic, a transparent switch learns from frame movements by inspecting the source address.

الشرح (عربي): لإنشاء جدول ديناميكي، يتعلم المحول الشفاف من تحركات الإطارات عن طريق فحص عنوان المصدر لمعرفة مكان المحطة.

--------------------------------------------------

Q6. What algorithm is used to prevent the loop problem in a switched LAN?

A) Link-state algorithm

B) Distance-vector algorithm

C) Spanning tree algorithm

D) Dijkstra's algorithm

Correct Answer: C

Explanation (En): The IEEE specification requires that switches use the spanning tree algorithm to create a loopless topology.

الشرح (عربي): تتطلب مواصفات IEEE من المحولات استخدام خوارزمية الشجرة الممتدة (Spanning tree algorithm) لإنشاء طوبولوجيا خالية من الحلقات.

--------------------------------------------------

Q7. What defines a virtual local area network (VLAN)?

A) A local area network configured by physical wiring.

B) A local area network configured by software.

C) A network connected across different cities using WANs.

D) A network that operates in the network layer only.

Correct Answer: B

Explanation (En): A VLAN is a local area network configured by software, not by physical wiring.

الشرح (عربي): الشبكة المحلية الافتراضية (VLAN) هي شبكة محلية يتم تكوينها بواسطة البرامج بدلاً من التوصيل المادي للأسلاك.

--------------------------------------------------

Q8. Which of the following is a characteristic used to group stations in a VLAN?

A) Interface numbers

B) MAC addresses

C) IP multicast addresses

D) All of the above

Correct Answer: D

Explanation (En): Vendors use different characteristics such as interface numbers, MAC addresses, IP addresses, or a combination of them.

الشرح (عربي): يستخدم البائعون خصائص مختلفة لتجميع المحطات في شبكات VLAN، مثل أرقام الواجهات، عناوين MAC، عناوين IP، أو مزيج منها.

--------------------------------------------------

Q9. Which IEEE standard defines the format for frame tagging in a VLAN?

A) IEEE 802.3

B) IEEE 802.11

C) IEEE 802.1d

D) IEEE 802.1Q

Correct Answer: D

Explanation (En): In 1996, the IEEE 802.1 subcommittee passed a standard called 802.1Q that defines the format for frame tagging.

الشرح (عربي): في عام 1996، أقرت لجنة IEEE 802.1 معياراً يسمى 802.1Q والذي يحدد التنسيق الخاص بوسم الإطارات (Frame tagging).

--------------------------------------------------

Q10. What is the primary duty of the network layer at the source host?

A) Error control for the whole datagram

B) Packetizing

C) Framing

D) Signal attenuation

Correct Answer: B

Explanation (En): The first duty of the network layer is packetizing: encapsulating the payload in a network-layer packet at the source.

الشرح (عربي): الواجب الأول لطبقة الشبكة هو الحزم (Packetizing)، أي تغليف البيانات في حزمة طبقة الشبكة عند المصدر.

--------------------------------------------------

Q11. What is the routing approach used by the Internet Protocol (IP) at the network layer?

A) Circuit switching

B) Connectionless service (Datagram approach)

C) Connection-oriented service (Virtual-circuit approach)

D) Hub broadcasting

Correct Answer: B

Explanation (En): The Internet network layer was designed to provide a connectionless service using the datagram approach.

الشرح (عربي): تم تصميم طبقة الشبكة في الإنترنت لتوفير خدمة غير متصلة (Connectionless service) باستخدام نهج مخطط البيانات (Datagram).

--------------------------------------------------

Q12. Which type of delay is calculated as the packet length divided by the transmission rate?

A) Propagation delay

B) Processing delay

C) Queuing delay

D) Transmission delay

Correct Answer: D

Explanation (En): The transmission delay is the time needed to put all bits on the line: Delay = (packet length) / (transmission rate).

الشرح (عربي): يُحسب تأخير النقل (Transmission delay) بقسمة طول الحزمة على معدل النقل.

--------------------------------------------------

Q13. How is throughput determined in a path with multiple links in series?

A) By the maximum transmission rate

B) By the average transmission rate

C) By the minimum transmission rate (the bottleneck)

D) By the sum of all transmission rates

Correct Answer: C

Explanation (En): The overall throughput is determined by the bottleneck, the pipe with the smallest diameter (minimum transmission rate).

الشرح (عربي): يتم تحديد الإنتاجية (Throughput) الكلية في مسار متعدد الروابط من خلال عنق الزجاجة (Bottleneck)، وهو الحد الأدنى لمعدل النقل في المسار.

--------------------------------------------------

Q14. What is the size of an IPv4 address?

A) 16 bits

B) 32 bits

C) 64 bits

D) 128 bits

Correct Answer: B

Explanation (En): An IPv4 address is a 32-bit address that uniquely and universally defines the connection of a host or a router.

الشرح (عربي): عنوان IPv4 هو عنوان يتكون من 32 بت يحدد بشكل فريد وعالمي اتصال المضيف أو الموجه بالإنترنت.

--------------------------------------------------

Q15. In classful addressing, which class was originally designed with an 8-bit prefix length?

A) Class A

B) Class B

C) Class C

D) Class D

Correct Answer: A

Explanation (En): In class A, the network length is 8 bits (with the first bit, 0, defining the class).

الشرح (عربي): في العنونة الفئوية (Classful addressing)، الفئة A لها طول شبكة يبلغ 8 بت (مع استخدام البت الأول لتحديد الفئة).

--------------------------------------------------

Q16. What was the main reason classful addressing became obsolete?

A) Too many classes to manage

B) Address depletion

C) Insufficient routing protocols

D) Poor security mechanisms

Correct Answer: B

Explanation (En): The reason that classful addressing has become obsolete is address depletion caused by inefficient distribution.

الشرح (عربي): السبب الرئيسي لإلغاء العنونة الفئوية هو استنفاد العناوين (Address depletion) بسبب التوزيع غير الفعال لها.

--------------------------------------------------

Q17. In classless addressing, what does the suffix of the IP address define?

A) The network block

B) The prefix length

C) The node (connection of a device to the Internet)

D) The subnet mask

Correct Answer: C

Explanation (En): The prefix in an address defines the block (network); the suffix defines the node (device connection).

الشرح (عربي): في العنونة غير الفئوية، تحدد البادئة (Prefix) الشبكة، بينما تحدد اللاحقة (Suffix) العقدة أو الجهاز المتصل.

--------------------------------------------------

Q18. What is the slash notation formally called in classless addressing?

A) Variable-length block routing

B) Classless interdomain routing (CIDR)

C) Address mask notation

D) Default routing notation

Correct Answer: B

Explanation (En): The slash notation is informally used and formally referred to as classless interdomain routing (CIDR).

الشرح (عربي): يُعرف تدوين الشرطة المائلة (Slash notation) رسمياً باسم التوجيه غير الفئوي بين النطاقات (CIDR).

--------------------------------------------------

Q19. Given the CIDR block 167.199.170.82/27, how many addresses are available in this network?

A) 16

B) 27

C) 32

D) 64

Correct Answer: C

Explanation (En): The number of addresses in the network is 2^(32-27) = 2^5 = 32.

الشرح (عربي): عدد العناوين في هذه الشبكة يُحسب بالمعادلة 2^(32-27) = 32 عنواناً.

--------------------------------------------------

Q20. How is the first address (network address) in a block found in classless addressing?

A) By keeping the n leftmost bits and setting the rightmost bits to 1s.

B) By setting all 32 bits to 0s.

C) By applying a bit-wise OR operation with the mask.

D) By keeping the n leftmost bits and setting the rightmost bits to 0s.

Correct Answer: D

Explanation (En): To find the first address, we keep the n leftmost bits and set the remaining (32 - n) rightmost bits to 0s.

الشرح (عربي): للعثور على العنوان الأول (عنوان الشبكة)، نحتفظ بالبتات n الموجودة في أقصى اليسار ونقوم بتعيين البتات المتبقية (32 - n) إلى أصفار (0).

--------------------------------------------------

Q21. What is the address mask in dotted-decimal notation for a prefix length of /24?

A) 255.0.0.0

B) 255.255.0.0

C) 255.255.255.0

D) 255.255.255.255

Correct Answer: C

Explanation (En): A /24 mask has 24 ones followed by 8 zeros, which translates to 255.255.255.0.

الشرح (عربي): يتكون قناع الشبكة /24 من 24 بت بقيمة (1) يليه 8 أصفار، مما يترجم إلى 255.255.255.0 بالنظام العشري المنقط.

--------------------------------------------------

Q22. What is the purpose of address aggregation in routing?

A) It increases the number of entries in the forwarding table.

B) It combines smaller blocks of addresses into a larger block to reduce the forwarding table size.

C) It fragments an IP datagram into smaller units.

D) It translates an IPv4 address to an IPv6 address.

Correct Answer: B

Explanation (En): Address aggregation combines blocks of addresses to create a larger block, routing based on the prefix of the larger block to save table space.

الشرح (عربي): تجميع العناوين (Address aggregation) يدمج كتل العناوين الصغيرة في كتلة أكبر لتقليل حجم جدول التوجيه.

--------------------------------------------------

Q23. Which protocol helps IPv4 handle and report errors in network-layer delivery?

A) ARP

B) IGMP

C) ICMPv4

D) DHCP

Correct Answer: C

Explanation (En): The Internet Control Message Protocol version 4 (ICMPv4) helps IPv4 to handle errors that may occur in delivery.

الشرح (عربي): يساعد بروتوكول (ICMPv4) الإصدار الرابع من بروتوكول الإنترنت (IPv4) في معالجة بعض الأخطاء والإبلاغ عنها أثناء توصيل البيانات.

--------------------------------------------------

Q24. In the IPv4 datagram format, what is the length of the base (main) header without options?

A) 10 bytes

B) 20 bytes

C) 40 bytes

D) 60 bytes

Correct Answer: B

Explanation (En): The IP header is 20 to 60 bytes in length. The first 20 bytes are the base (main) header.

الشرح (عربي): يتراوح طول ترويسة IP من 20 إلى 60 بايت. وتشكل أول 20 بايت الترويسة الأساسية.

--------------------------------------------------

Q25. How is the total length of the datagram header represented in the HLEN field of an IPv4 packet?

A) In bits

B) In bytes

C) In 4-byte words

D) In 8-byte words

Correct Answer: C

Explanation (En): The 4-bit header length (HLEN) field defines the total length of the datagram header in 4-byte words.

الشرح (عربي): يحدد حقل HLEN المكون من 4 بتات الطول الإجمالي لترويسة مخطط البيانات بوحدات تتكون من 4 بايت (4-byte words).

--------------------------------------------------

Q26. What is the purpose of the time-to-live (TTL) field in an IPv4 datagram?

A) To record the exact time the datagram was sent

B) To control the maximum number of hops a datagram can visit

C) To define the total length of the payload

D) To set the priority of the packet

Correct Answer: B

Explanation (En): The TTL field is used to control the maximum number of hops (routers) visited by the datagram to prevent it from looping indefinitely.

الشرح (عربي): يُستخدم حقل (TTL) للتحكم في الحد الأقصى لعدد القفزات (الموجهات) التي يمكن أن يزورها مخطط البيانات لمنع الدوران اللانهائي.

--------------------------------------------------

Q27. Which field in the IPv4 header is responsible for multiplexing and demultiplexing the payload?

A) Identification

B) Fragmentation offset

C) Service type

D) Protocol

Correct Answer: D

Explanation (En): The protocol field defines to which upper-layer protocol the payload should be delivered, providing multiplexing and demultiplexing.

الشرح (عربي): يوفر حقل البروتوكول (Protocol) دمج الإشارات عند المصدر وفصلها عند الوجهة لتحديد بروتوكول الطبقة العليا المستقبل (مثل TCP أو UDP).

--------------------------------------------------

Q28. Does the header checksum in an IPv4 datagram check the payload for errors?

A) Yes, it checks both the header and the payload.

B) No, it checks only the header.

C) No, it checks only the payload.

D) It only checks the destination IP address.

Correct Answer: B

Explanation (En): IP puts the burden of error checking of the payload on the protocol that owns it. The checksum field only checks the IP header.

الشرح (عربي): يقتصر حقل المجموع الاختباري (Checksum) في IPv4 على فحص الترويسة (Header) فقط ولا يتحقق من وجود أخطاء في حمولة البيانات (Payload).

--------------------------------------------------

Q29. What does the Maximum Transfer Unit (MTU) refer to?

A) The maximum size of an IP datagram (65,535 bytes).

B) The maximum size of the payload that can be encapsulated in a frame for a specific physical network.

C) The maximum number of fragments an IP datagram can be divided into.

D) The maximum length of an IPv4 header.

Correct Answer: B

Explanation (En): MTU is the maximum size of the payload that can be encapsulated in a frame for a particular physical link-layer protocol.

الشرح (عربي): وحدة النقل القصوى (MTU) هي الحد الأقصى لحجم حمولة البيانات التي يمكن تغليفها في إطار (Frame) لبروتوكول شبكة مادية معينة.

--------------------------------------------------

Q30. Which field in the IPv4 header helps the destination correctly assemble fragmented IP datagrams?

A) Service type

B) Checksum

C) Identification

D) Options

Correct Answer: C

Explanation (En): The identification number helps the destination in reassembling the datagram. All fragments from the same datagram have the same identification number.

الشرح (عربي): يساعد رقم التعريف (Identification) الوجهة في إعادة تجميع مخطط البيانات، حيث تحمل جميع الأجزاء لنفس المخطط نفس رقم التعريف.

--------------------------------------------------

Q31. In IP fragmentation, what units are used to measure the value of the fragmentation offset?

A) Bits

B) Bytes

C) 4-byte units

D) 8-byte units

Correct Answer: D

Explanation (En): The 13-bit fragmentation offset field shows the relative position measured in units of 8 bytes.

الشرح (عربي): يوضح حقل إزاحة التجزئة (Fragmentation offset) الموقع النسبي للجزء، ويُقاس بوحدات تتكون من 8 بايت.

--------------------------------------------------

Q32. What does an M bit (more fragment bit) value of 0 indicate in an IPv4 datagram?

A) It is the first fragment of many.

B) The datagram must not be fragmented.

C) It is the last or only fragment.

D) There are more fragments following this one.

Correct Answer: C

Explanation (En): If the M bit value is 0, it means there are no more fragments; this is the last or only fragment.

الشرح (عربي): إذا كانت قيمة بت M (المزيد من الأجزاء) هي 0، فهذا يعني أن هذا هو الجزء الأخير أو الجزء الوحيد من المخطط.

--------------------------------------------------

Q33. Which of the following is an IPv4 single-byte option?

A) Record route

B) Strict source route

C) Timestamp

D) No operation

Correct Answer: D

Explanation (En): There are two single-byte options in IPv4: 'no operation' and 'end of option'.

الشرح (عربي): هناك خياران يتكونان من بايت واحد في IPv4: 'بدون عملية' (No operation) و 'نهاية الخيار' (End of option).

--------------------------------------------------

Q34. Which security attack involves an intruder passively making a copy of an IP packet without modifying it?

A) Packet modification

B) IP spoofing

C) Packet sniffing

D) Denial of service

Correct Answer: C

Explanation (En): Packet sniffing is a passive attack in which the attacker intercepts and makes a copy of the packet secretly.

الشرح (عربي): استنشاق الحزم (Packet sniffing) هو هجوم سلبي يقوم فيه المهاجم باعتراض الحزمة وعمل نسخة منها دون تعديلها.

--------------------------------------------------

Q35. What protocol provides a connection-oriented secure service for IP packets?

A) ICMPv4

B) IGMP

C) IPSec

D) ARP

Correct Answer: C

Explanation (En): IPSec (IP Security) creates a secure connection-oriented service between two entities exchanging IP packets.

الشرح (عربي): يقوم بروتوكول IPSec (أمان IP) بإنشاء خدمة آمنة موجهة للاتصال بين كيانين يتبادلان حزم IP.

--------------------------------------------------

Q36. Which ICMPv4 error message is sent to slow down the sending of more datagrams due to congestion?

A) Destination Unreachable

B) Source Quench

C) Redirection

D) Time Exceeded

Correct Answer: B

Explanation (En): The source quench message informs the sender that the network has encountered congestion and datagrams are being dropped.

الشرح (عربي): تُعلم رسالة (Source Quench) المرسل بوجود اختناق في الشبكة وتطلب منه تخفيف سرعة إرسال المزيد من مخططات البيانات.

--------------------------------------------------

Q37. Which ICMPv4 error message is generated when a datagram's Time-to-Live (TTL) reaches 0?

A) Parameter Problem

B) Destination Unreachable

C) Source Quench

D) Time Exceeded

Correct Answer: D

Explanation (En): When the TTL value becomes 0, the datagram is dropped by the router and a time-exceeded message is sent.

الشرح (عربي): عندما تصل قيمة (TTL) إلى الصفر، يتم إسقاط مخطط البيانات وإرسال رسالة تجاوز الوقت (Time Exceeded).

--------------------------------------------------

Q38. How large is the address space for an IPv6 address?

A) 32 bits

B) 64 bits

C) 128 bits

D) 256 bits

Correct Answer: C

Explanation (En): Internet Protocol version 6 (IPv6) uses a 128-bit address space.

الشرح (عربي): يستخدم الإصدار السادس من بروتوكول الإنترنت (IPv6) مساحة عنونة واسعة بحجم 128 بت.

--------------------------------------------------

Q39. Which address category is explicitly NOT supported by IPv6?

A) Unicast

B) Multicast

C) Anycast

D) Broadcast

Correct Answer: D

Explanation (En): IPv6 does not define broadcasting; it considers broadcasting as a special case of multicasting.

الشرح (عربي): لا يدعم بروتوكول IPv6 عناوين البث العام (Broadcast)، بل يعتبره حالة خاصة من البث المتعدد (Multicast).

--------------------------------------------------

Q40. What is the fixed length of the IPv6 base header?

A) 20 bytes

B) 40 bytes

C) 60 bytes

D) Variable length

Correct Answer: B

Explanation (En): An IPv6 packet is made up of a base header that is fixed at exactly 40 bytes.

الشرح (عربي): تتكون حزمة IPv6 من ترويسة أساسية ثابتة بحجم 40 بايت وبعض ترويسات التمديد (Extension headers).

--------------------------------------------------

Q41. In IPv6, which technique replaces the options fields originally found in the IPv4 base header?

A) Extension headers

B) Payload padding

C) Fixed base header

D) Flow labels

Correct Answer: A

Explanation (En): To give more functionality, the IPv6 base header can be followed by up to six extension headers, replacing the options in IPv4.

الشرح (عربي): لإعطاء المزيد من الوظائف، يمكن إتباع الترويسة الأساسية في IPv6 بما يصل إلى ست ترويسات تمديد (Extension headers) والتي تحل محل الخيارات في IPv4.

--------------------------------------------------